DESKTOP & NOTEBOOK IDE/ATAPI CD-ROM DRIVES

MS-DOS DRIVER INSTALLATION GUIDE

This installation guide is divided into the following sections:

SECTION 1: IMPORTANT DRIVER NOTE. (PLEASE READ!)

# SECTION 2: Pre-installation checklist

# SECTION 3: Installing the MS-DOS mode driver

SECTION 4: Creating a bootable diskette

# SECTION 5: Contacting TEAC’s TECHNICAL SUPPORT SERVICES

Back to top

SECTION 1: Important driver note. (PLEASE READ!)

NOTE: For the most part, this document references a desktop IDE CD-ROM drive, but the general software driver instructions also apply to Notebook PC CD-ROM drive models.

All TEAC models have a standard IDE/ATAPI interface, and TEAC America provides only the software driver for MS-DOS/Windows 3.1x.

The ATAPICD.EXE is available from our web site. This executable file has a self-extracting mechanism that provides the TEAC_CDI.SYS and this document (IDE_DOS.DOC). You may download this program from our web site at any time: http://www.teac.com/dsp/cd/dtide_d.html

In operating systems such as Windows 95, Windows 98, Windows NT, and OS/2 Warp, you will be using the built-in IDE\ATAPI driver in the corresponding operating system software.

Back to top

## SECTION 2: Pre-installation checklist

Please follow these steps to insure a proper installation of your CD-ROM drive:

IDENTIFYING THE IDE PORT

NOTE: For Notebook PC models, please reference to your Notebook PC manufacturer's installation instructions.

As an industry standard, the IDE ports in AT-Compatible desktop PCs have been assigned an identifiable sequence.

The sequence is as follows:

PRIMARY

SECONDARY

###### TERTIARY

QUATERNARY

In most systems, there will be either one or two IDE ports (Primary and Secondary). You find both ports on Interface Controller Cards, and embedded ports in the motherboard. IDE hard drives are always connected to the PRIMARY IDE PORT. On systems with only one IDE port (Primary), you need to add an IDE interface card, and the IDE port on the card will have to be configured as the SECONDARY port. If you are currently using both IDE ports, then your interface card or the IDE port in your soundcard must be configured as TERTIARY. Each IDE port can handle up to two IDE devices (Master and Slave).

NOTE: Industry-wide, it is not recommended to daisy chain either a CD-ROM or TAPE BACKUP drive with your hard disk drive. The performance of the hard disk drive will be degrated to the maximum speed of the other device since the data transfer rate is as fast as the transfer rate of the slowest device. The slowest device in this case will be either the CD-ROM or TAPE BACKUP drive.

It is recommended to connect your desktop CD-ROM drive as a MASTER in the next available IDE port in your system.  Please reference to your system’s manual to identify the number of IDE ports available in your machine.

You can guide yourself by identifying where in the system your hard disk drive C: is physically connected to. In older style PCs, normally you will find the hard disk drive connected to an Interface Card. Check to see if there is a second (unused) 40-pin, male IDE connector on that card. On most current PCs, you will find the hard disk drive connected directly into the motherboard. Is there another 40-pin IDE connector on the motherboard?

You have the following options:

- If there is an unused IDE port on the controller card or embedded within the motherboard, connect the CD-ROM DRIVE to that IDE port. This offers two advantages:  First, you will not have to use another slot on the

motherboard for an additional IDE card. Second, the secondary IDE connector already available in your

system may be an Enhanced IDE port which will definitely provide faster data transfer through the IDE bus.

- If there is no other IDE port in your system except the one which your hard disk drive is connected to, connect the CD-ROM drive to a soundcard with an IDE port or a Secondary IDE controller card.

ENABLING THE IDE PORT

Please make sure the IDE port you select to connect your desktop CD-ROM drive is enabled or activated.  Please reference your controller card or soundcard’s documentation for the proper procedure to enable the IDE port, for it may be done through software or hardware (jumper or dipswitch settings). For example, on most current systems, the IDE ports may be enabled or disabled through the CMOS Setup System configuration. If the IDE port being used to connect the CD-ROM drive is not enabled, your drive may appear as though is not even connected to the system and subsequent attempts for loading of any software drivers WILL BE UNSUCCESSFUL.

CD-ROM DRIVE MASTER\SLAVE CONFIGURATION

Make sure the CD-ROM drive is properly configured for Master or Slave. Please reference the CD-ROM drive's installation guide for the proper configuration instructions.

- If you find it necessary to connect the desktop CD-ROM drive in daisy-chain with your hard disk drive,

configure the CD-ROM drive as SLAVE. Please note that some hard disk drives may have different

configuration setting for a Single as compared to a Master in a dual-drive configuration. Therefore, in some

cases when you daisy chain the CD-ROM drive to your hard disk drive, it may also be necessary to change

the hard disk drive’s configuration from a single to a Master drive configuration.

NOTE: There are some hard disk drive manufacturers that do not allow a CD-ROM drive to be connected as a Slave to their units. Please reference to your HDD documentation if necessary.

- If you have the CD-ROM drive connected by itself on any of the available IDE ports, the drive has to be

configured as MASTER.

Back to top

SECTION 3: Installing the MS-DOS mode driver

RUNNING THE CD-ROM DRIVE IN THE MS-DOS MODE

In order for the CD-ROM drive to properly work in the MS-DOS environment, you need the software driver TEAC_CDI.SYS. This software driver comes from TEAC on the White and Blue labeled installation diskette for the CD-ROM DRIVE. If you do not have such diskette, please reference to section 1 for information on how to obtain this driver.

The MSCDEX.EXE ver 2.23 or above is required for a successful installation. This driver can be found in the C:\DOS directory in any machine running MS-DOS 6.0 and above, and in the C:\WINDOWS\COMMAND in Windows 95/98 machines.

DRIVER INSTALLATION UNDER MS-DOS (ONLY) ENVIRONMENT

In order for the CD-ROM drive to properly work in the MS-DOS environment, you need to run the INSTALL.EXE program. This software comes with the TEAC White and Blue labeled installation diskette for the CD-ROM drive. If you do not have such diskette, please reference to section 1 for information on how to obtain the MS-DOS driver.

- Insert TEAC diskette in to drive A: and type  A:\INSTALL.

- Choose AUTO SELECT or SELECT I/O PORT.

- If choosing SELECT I/O PORT, please refer to section 2 for your CD-ROM configuration.

- Choose MODIFY YOUR AUTOEXEC.BAT.

- Choose MODIFY YOUR CONFIG.SYS.

- Remove your INSTALLATION DISK from your floppy disk drive.

- Reboot your computer.

The installation is now complete.

INSTALLING THE TEAC_CDI.SYS FROM THE ATAPICD.EXE

- Create the TEAC directory and save the ATAPICD.EXE in this TEAC directory

- Change directory to TEAC by typing CD\TEAC and hit enter

- Type ATAPICD and hit enter. Two files will be decompressed: TEAC_CDI.SYS and IDE_DOS.DOC.

- Go back to the root directory by typing CD\ and hit enter

- Edit the Config.sys file. The procedure for editing the Config.sys is as follows:

At the C:\ prompt, type “edit Config.sys” and hit enter, then manually enter the following:

If you have the CD-ROM drive connected to the primary or secondary IDE port, edit this statement:

DEVICE=C:\TEAC\TEAC_CDI.SYS /D:TEAC-CDI

If you have the CD-ROM drive connected to the tertiary IDE port, edit this statement:

DEVICE=C:\TEAC\TEAC_CDI.SYS /D:TEAC-CDI /P:T-X

If you have the CD-ROM drive connected to the quaternary IDE port, edit this statement:

DEVICE=C:\TEAC\TEAC_CDI.SYS /D:TEAC-CDI /P:Q-X

- Edit the AUTOEXEC.BAT FILE and add the following statement:

C:\DOS\MSCDEX.EXE /D:TEAC-CDI /M:15

Save both files and reboot your system.

# DRIVER INSTALLATION UNDER WINDOWS 3.1x USING THE INSTALL.EXE FROM THE FLOPPY

# DISK

- In Windows, go to the FILE MANAGER in the MAIN Windows group.

- Select the drive in which the TEAC CD-ROM drive installation diskette is installed.

- Double-click on the INSTALL.EXE file to execute the setup program.

- Select the directory where TEAC software is to be installed in. (TEAC is the default directory name)

- On the next screen, if you have the CD-ROM drive connected to the PRIMARY or SECONDARY IDE ports, choose AUTO SELECT and go to the next step.  If you have the CD-ROM drive to a TERTIARY or QUATERNARY IDE ports, choose SELECT I/O PORT for which in the next two screens you will have to select the IDE port the drive is connected to add its MASTER\SLAVE configuration.

- The software should now be loaded in the specified directory.

- Select YES to have a “TEAC CD-PLAYER” group icon created in the Program  Manager

- Select OK so that both the Config.sys and Autoexec.bat files are automatically edited.

- The installation is complete.

- Reboot your system.

NOTE: The device driver called “MCI CD AUDIO” is required to use the Windows 3.1x version of the TEAC AUDIO-CD PLAYER.

- Go to Control Panel in the Main group.

- Click on Drivers.

- Under the Installed driver’s window, Click on the button labeled ADD.

- Under the Drivers to be Installed window, Double-click on the “MCI AUDIO CD” driver.

- Insert the Windows 3.x diskette you’re being prompt to and the driver will be installed.

- Reboot the system to ensure the driver’s loaded properly.

- You should now have full operation of the TEAC AUDIO-CD PLAYER.

INSTALLATION UNDER MS-DOS AND WINDOWS 95

As mentioned above, the INSTALL.EXE routine will not work in a system running MS-DOS and Window 95, therefore, to properly install the MS-DOS driver, your have to manually install it as per the steps below.

The procedure for editing the Config.sys is as follows:

At the C:\ prompt, create a directory named TEAC. Copy into the newly created TEAC directory the file TEAC_CDI.SYS from the White and Blue labeled diskette you received with your drive or from the decompressed ATAPICD.EXE file you downloaded from our website.

At the C:\ prompt, manually by typing “edit Config.sys” and hit enter:

If you have the CD-ROM drive connected to the primary or secondary IDE port, edit this statement:

DEVICE=C:\TEAC\TEAC_CDI.SYS /D:TEAC-CDI

If you have the CD-ROM drive connected to the tertiary IDE port, edit this statement:

DEVICE=C:\TEAC\TEAC_CDI.SYS /D:TEAC-CDI /P:T-X

If you have the CD-ROM drive connected to the quaternary IDE port, edit this statement:

DEVICE=C:\TEAC\TEAC_CDI.SYS /D:TEAC-CDI /P:Q-X

In Windows 95, the CD-ROM extension statement is no longer edited in the AUTOEXEC.BAT, therefore, to use CD-ROM drive in MS-DOS mode, edit the DOSSTART.BAT file located in Windows directory and add the CD-ROM extension statement as follows:

C:\WINDOWS\COMMAND\MSCDEX.EXE /D:TEAC-CDI /M:15

- Save the changes made to the DOSSTART.BAT file.

- Bootup the system

- When you see the message “Starting Windows 95”, press the F8 key.

- At the Startup menu, select “Command Prompt Only”.

- You should now have access to the CD-ROM DRIVE CD-ROM drive in the DOS mode.

- To get back to Windows, at the DOS prompt, type the word “EXIT” and press ENTER.

DEVICE= <drv:\path\>TEAC_CDI.SYS  /D:<xxxxxxxx>  /P:<xxx>

| SWITCH | Description |
| --- | --- |
| /D:<xxxxxxxx> | This switch name specifies the device name which must be the SAME as in the mscdex.exe line in the autoexec.bat or DOSSTART.BAT file. |
| /P:<xxx> | This switch specifies the IDE port the CD-ROM DRIVE is connected to. It also specifies whether the CD-ROM DRIVE detected as the MATER or SLAVE device. For example: /P:T-M shows the CD-ROM DRIVE connected to the TERTIARY port as a MASTER. |

Back to top

SECTION 4: Creating a bootable diskette

- Turn on your computer and get to a DOS prompt.  If your computer starts up in Windows 3.1x you  will then  need to exit to DOS.

- Once you are at a DOS prompt C:\ insert a blank disk into your A: drive.

- Type FORMAT A: /S and press enter

- After the format is finished, change drives to you’re a: drive and type Edit Config.sys and press enter

- You now need to type the following  two lines in your Config.sys file:

If you have the CD-ROM drive connected to the primary or secondary IDE port, edit this statement:

DEVICE=A:\TEAC_CDI.SYS /D:TEAC-CDI

If you have the CD-ROM drive connected to the tertiary IDE port, edit this statement:

DEVICE=A:\TEAC_CDI.SYS /D:TEAC-CDI /P:T-M

If you have the CD-ROM drive connected to the quaternary IDE port, edit this statement:

DEVICE=A:\TEAC_CDI.SYS /D:TEAC-CDI /P:Q-M

The lastdrive statement is also needed:

#### LASTDRIVE=Z

- Save the Config.sys file and now create the AUTOEXEC.BAT file

- Open the File menu and select New, at the open box, type the filename AUTOEXEC.BAT and hit enter

- Type the following line in the file:

##### A:\MSCDEX.EXE /D:TEAC-CDI /M:15

- Save the Autoexec.bat file and exit to the MS-DOS Prompt.

- The next step is to copy the following two files to the root of A:\ on your Bootable Disk.

TEAC_CDI.SYS: Which is located on the TEAC blue and white Installation Disk, OR from the ATAPICD.EXE you just downloaded. Please refer to section 1 for download information.

MSCDEX.EXE: This file is located in the C:\DOS directory in any machine running DOS 6.0 or above. It is also found in the C:\WINDOWS\COMMAND folder in any Windows 95/98 machine.

Once you have both files in your Bootable Disk, you will be able to bootup off the floppy drive and have access to the Desktop or Notebook PC IDE CD-ROM drive.

Back to top

SECTION 5: Contacting TEAC’s TECHNICAL SUPPORT SERVICES

## TEAC TECHNICAL SUPPORT SERVICES

TEAC Support Services may be reached from Monday through Friday, except for holidays, from 8:00 a.m. to 5:00 p.m. Pacific Time.

## TECHNICAL SUPPORT

For technical questions and comments, please contact Technical Support Group.

(323) 727-4860 VOICE

(323) 869-8751 FAX

DSPDTSG@TEAC.COM EMAIL

## SERVICE DEPARTMENT

You may contact our Service Department for product warranty and service information, including issuance of RMAs.

(323) 726-0303 Ext. 616 and 694 VOICE

(323) 727-7606 FAX

DSSVCE@TEAC.COM EMAIL

## PARTS DEPARTMENT

Please contact our Parts Department for availability of parts and manuals.

(323) 727-4840 VOICE

(323) 727-7632 FAX

PARTS@TEAC.COM EMAIL

SALES DEPARTMENT

If you are interested in single unit purchases i.e. where to buy, pricing, availability, etc. please contact our headquarters office.

(323) 726-0303 VOICE

(323) 727-7652 FAX

DSSALES@TEAC.COM EMAIL

## AUTOMATIC SUPPORT

Please visit our Web site for automatic support, technical documentation, drivers and firmware downloads, and FAQs.

HOME PAGE: http://www.teac.com/dsp/new-dsp.html

The Fax on Demand (24-hr.-a-day, 7-days-a-week) system provides technical documents

for all of our products. The technical documentation available includes installation guides, product spec sheets, and jumper setting information, as well as troubleshooting guides. From a touch-tone phone, call into the Fax on Demand System to have a catalog of available documents faxed to you (Document 1002), then select a specific document from the catalog. All the required information, document number and your fax machine number, is input through your touch-tone phone’s keypad.

(323) 727-7629 FAX ON DEMAND

Back to top

Copyright © 1999 by TEAC America, Inc.

All trademarks or product names mentioned herein are the property of their respective owners.